A Fixed Functional Appliance

D.J. Spary

Clinical Visit 1

Separators mesial and distal to upper 6/6 lower 6/6.

Clinical  Visit 2

Remove separators and select bands slightly too large. (Make a note of the size of the bands.) Take two upper and two lower impressions one with the bands in place and one with the bands removed. Carefully place the bands in the impression and cement a second set of bands on the teeth to maintain the spaces. (If available it is better to use a poly-carboxylate cement such as “Poly F” as this makes it easier to remove the cement from the molars at the next visit.)

Measure the overjet at maximum posture forwards in most cases the fixed functional should be made to a 2mm overjet but if the patient cannot achieve this the functional appliance should be made to the most forward position.

Note the overbite in cases with an increased overbite the lower part of the appliance will need acrylic over the lower 6 and 5 but this is not required in open bite cases.

If a crossbite develops when the patient postures forwards to a normal overjet warn the laboratory that you may wish to slightly expand the upper and contract the lower. The hangers should be placed a little palataly.

Lab work

Cast the impressions with the bands in these will be the working models.

Cast the other impressions to give the spare working models for use if the working models are damaged during soldering.

Duplicate these models or recast impressions for study models

Bend the Trans Palatal Arch (TPA) in 1.00mm or 1.25mm steel note the extra loops for firm retention of the acrylic.

Bend the tags from upper 4/4.

Solder TPA. If the model is too badly damaged during soldering transfer the TPA to the spare working model.

Wax the tags to the upper 4/4.

In the lower arch bend the lingual arch (Australian wire 0.9mm)

Solder the lingual arch to the lower molar bands. If the model is badly damaged during soldering, transfer to the spare model.

Place models in a plaster-less articulator and unless specified on the lab sheet, set to a normal overjet with coincident centrelines.

Bend the hangers in 1.5mm wire. Although the height of the hangers depends on the overbite and the slope of the palate the hangers for this type of fixed functional are about 4mm shorter than those for a Dynamax.

Initially set the hangers in place with Ortho-care Block-Out Compound (from Ortho-Care Limited, 5 Oxford Place Bradford, West Yorkshire BD3 0EF. Cat No 9001-220). This is a non setting material similar to Blu-Tack. Check the position is just behind the circle hooks on the lower appliance.

If the alignment is satisfactory remove the block out compound and coat acrylic onto the prepared models. (Models should be prepared by soaking in water for 20 minutes) Place the hangers into the acrylic and check alignment with the lower using the articulator. It is easier to do this with the articulator upside down. Because the acrylic has not set it is possible to finalise the position of the hangers.

Process the upper appliance on the articulator with the lower in position. Process for 15 minutes in a hydroflask.

Build up the acrylic in the upper appliance

Build up acrylic button in the lower. If the overbite is increased the acrylic can be extended over lower 5/5 and the mesial part of lower 6/6 leaving enough space to fit band removing pliers.

When the buttons are complete check that the overjet is 2mm and that the centrelines are coincident/

Polish the appliance and check that the tubes are clear.

Clinical visit 3

Remove the molar bands and clean the teeth.

Fit the fixed functional and check the overjet and centrelines.

Check the overbite. If the covering over lower 5/5 causes an anterior open bite, thin down the acrylic.

Check the hangers are in the correct position, if they are too lingual they can be bent palataly but you need to hold the acrylic firmly because they are made of very thick wire.

If posturing the mandible causes the development of a cross bite there is a little scope for expanding the upper and contracting the lower. (It is best to warn the technician if you plan to do this and the hangers can be moved a fraction more palataly.)

Cement the upper appliance.

Etch the occlusal surface of upper 4/4 for at least 30 seconds then bond the rests to the occlusal surface with a thick layer of composite.

Problems.

- Hangers too wide. (Hold the acrylic firmly and bend them in a little.)

- Patient can not bite forwards far enough. (Trim back the buttons on the lower making sure that you maintain the centreline.)

- Overjet still too big. (Build up the lower buttons with a little cold cure acrylic or flowable composite.)

Warning to patients.

This appliance does not break easily but it can become de-cemented. If this happens they must contact and have the appliance stuck back in firmly or the molar bands will become distorted. Use good quality bands and cement.

Visit 4

Check progress record overjet on maximum forward posture.

If you want to advance the lower jaw further you can add flowable composite to the lower button or remove the lower appliance and build it up with cold-cure acrylic.

Visit 5

Full straight-wire bond up.

How long should you continue with the fixed functional?

The patients can usually achieve a reverse overjet of 5 mm after 4 months. After 6 months remove the appliance and cement new molar bands. As you have noted the size (see visit 2) the nurse will be able to get them ready in advance.

What about extractions?

In most cases it is best to treat to a normal overjet and overbite non extraction and then have a lateral skull X-ray. If there is an element of bi-maxillary protrusion then consider extraction.